# eFloStop II Wi-Fi Hub — Cloud-to-Device Command Catalogue

Every command the cloud can send, as a real example  ·  v1.0

| Document version | 1.0 |
| --- | --- |
| Firmware version | 1.9.0 [CMakeLists.txt:12] |
| Git commit | f852ddf7b4209db1fbd2539babcb791898eed94a |
| Schema | eflostop.cmd |
| Direction | cloud → hub (C2D) |
| Transport | devices/<device_id>/messages/devicebound/# (MQTT, QoS 1) |
| Command count | 11 commands across 4 groups |

This document lists every command the cloud can send to the eFloStop II Wi-Fi Hub. Each entry is a
complete message exactly as it goes on the wire, followed by the acknowledgement you get back and every rejection
the hub can answer with. The error text is quoted exactly as the firmware emits it.

Commands arrive as ordinary Azure IoT Hub cloud-to-device messages. There are no direct methods and no
device-twin commands; the twin carries configuration only. The hub subscribes once on connect and dispatches
every message it receives on that topic.

Read the Traps section at the end before you write the client. Several behaviours are surprising enough to
cost a day if you meet them in the field instead of here — in particular, a command with the wrong schema can
still execute while sending you no acknowledgement at all.

# Contents

Update field

Word fills this in when fields are updated: select all (Ctrl+A) then press F9.

# The command envelope

Every command is a JSON object with the same four top-level keys. payload is present only
for commands that take arguments.

Keys are read in the order shown. The parser is strict about schema and cmd and permissive about everything
else — ver in particular is read and then never checked.

{
  "schema": "eflostop.cmd",
  "ver": 1,
  "cmd": "valve_close",
  "id": "req-8f23a1"
}

| Key | Type | Required | Notes |
| --- | --- | --- | --- |
| schema | string | yes | Exactly "eflostop.cmd" (or the accepted legacy "eflostop.cmd.v1"). Compared case-sensitively. Any other value does NOT produce an error — see Trap 1. |
| ver | number | no | Defaults to 1 when absent. Read, logged, and then ignored: there is no range check and nothing branches on it. Any number is accepted. |
| cmd | string | yes | The command name, matched case-sensitively. Truncated to 31 characters without warning; a longer name therefore matches nothing and answers "unknown command". |
| id | string | no | Your correlation id, echoed back on the acknowledgement. Truncated to 63 characters without warning. Omit it and you still get an ack for an enveloped command, but the ack carries no id key at all — it is absent, not empty. |
| payload | object | conditional | Required by the six commands that take arguments; must be absent or ignored for the other five. |

# Acknowledgements

The hub answers with a cmd_ack. This is not a reply channel — it is an ordinary telemetry event
on the normal device-to-cloud topic, so you correlate it by data.id.

An ack is sent when the command arrived as a JSON envelope, or when it carried a non-empty id. A legacy
plain-text command satisfies neither, so it executes silently. data.error.code is the command name rather than
a machine-readable code, so match on data.error.detail if you need to distinguish causes.

One caution on correlation: when you omit id, the ack omits data.id entirely rather than sending an empty
string, so a consumer that requires the key will fail to parse.

Success:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 3600
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-8f23a1",
    "cmd": "valve_close",
    "status": "ok"
  }
}

Failure:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 3612
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-9c02b7",
    "cmd": "valve_open",
    "status": "error",
    "error": {
      "code": "valve_open",
      "detail": "Valve is locked after a leak (RMLEAK). Clear it with leak_reset first, or use override to open the valve during a leak."
    }
  }
}

# Index of commands

| ID | cmd | Payload | What it does |
| --- | --- | --- | --- |
| V1 | valve_open | none | Open the valve |
| V2 | valve_close | none | Close the valve |
| V3 | valve_set_state | yes | Set the valve to a given position |
| L1 | leak_reset | none | Clear the leak latch after the leak is fixed |
| L2 | override_enable | none | Start a 24-hour water-access override |
| L3 | override_cancel | none | Cancel the override window early |
| P1 | provision | yes | Commission the hub |
| P2 | decommission | yes | Remove one device, or wipe the hub |
| C1 | sensor_meta | yes | Name a sensor and give it a location |
| C2 | rules_config | yes | Configure auto-close |
| C3 | set_hub_name | yes | Name the hub |

# Valve control

Three commands, two code paths. valve_set_state routes to exactly the same handlers as valve_open and valve_close, so prefer it for new work: one command, one shape, and the state you want is explicit rather than implied by the command name. All three queue a BLE write and return immediately — the ack means accepted, not the valve moved. Watch for the valve_state_changed event to know it actually moved.

## V1 — valve_open

Queues a BLE write to open the valve. Refused while the post-leak interlock (RMLEAK) is set — that is the only refusal path. There is no provisioning check: an uncommissioned hub accepts this and queues a connect that will never match.

{
  "schema": "eflostop.cmd",
  "ver": 1,
  "cmd": "valve_open",
  "id": "req-open-001"
}

The ack means the write was queued, not that the valve moved — the return value of the queue post is not checked, so a full command queue is dropped silently and still acked ok. Opening an already-open valve acks ok and emits no event, because valve_state_changed is delta-gated.

Acknowledgement on success:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 3600
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-open-001",
    "cmd": "valve_open",
    "status": "ok"
  }
}

Rejections. Each is a cmd_ack with status:"error"; the error.detail text below is exact.

| error.detail | When |
| --- | --- |
| Valve is locked after a leak (RMLEAK). Clear it with leak_reset first, or use override to open the valve during a leak. | The valve's RMLEAK interlock is asserted, i.e. it latched after a leak. Clear it with leak_reset, or use override_enable to get water during a live leak. |

main/iothub/app_iothub.c:657-667; refusal main/iothub/app_iothub.c:536-543

## V2 — valve_close

Queues a BLE write to close the valve. Unconditional — this command has no rejection path and can only ever ack ok.

{
  "schema": "eflostop.cmd",
  "ver": 1,
  "cmd": "valve_close",
  "id": "req-close-001"
}

Does not touch RMLEAK, the leak incident latch or an active override window — closing the valve during an override window does not cancel the window.

Acknowledgement on success:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 3600
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-close-001",
    "cmd": "valve_close",
    "status": "ok"
  }
}

main/iothub/app_iothub.c:668-672

## V3 — valve_set_state

The unified form, and the one to prefer. Routes to exactly the same code as V1/V2 depending on the value, including the same RMLEAK refusal when opening.

{
  "schema": "eflostop.cmd",
  "ver": 1,
  "cmd": "valve_set_state",
  "id": "req-state-001",
  "payload": {
    "state": "closed"
  }
}

| Key | Type | Required | Rules |
| --- | --- | --- | --- |
| state | string | yes | Exactly "open" or "closed", case-sensitive. No aliases, no trimming. "OPEN", "Open", "close" and "shut" are all rejected. |

Unlike valve_open and valve_close, this command has no legacy plain-text form — and that turns out to be a safety property. A wrong schema on this command is rejected outright, whereas the same mistake on V1/V2 executes silently (Trap 1).

Acknowledgement on success:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 3600
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-state-001",
    "cmd": "valve_set_state",
    "status": "ok"
  }
}

Rejections. Each is a cmd_ack with status:"error"; the error.detail text below is exact.

| error.detail | When |
| --- | --- |
| missing 'state' field (expected "open" or "closed") | No payload, payload is not valid JSON, `state` is absent, or `state` is not a string. Checked before the value is looked at. |
| Valve is locked after a leak (RMLEAK). Clear it with leak_reset first, or use override to open the valve during a leak. | state is "open" and the RMLEAK interlock is set. Same refusal as V1, but error.code here is "valve_set_state". |
| invalid state value (expected "open" or "closed") | `state` is a string but is neither exactly "open" nor exactly "closed". Checked last. |

main/iothub/app_iothub.c:674-699

# Leak and override

These manage the post-leak interlock. leak_reset is the normal path once the leak is fixed; override_enable is the sanctioned way to get water while a leak is still live. Note that override_enable can block for up to 10 seconds while it reconnects to the valve.

## L1 — leak_reset

Clears the latched incident and the valve's RMLEAK interlock, cancelling any active override window, and restores normal auto-close. Deliberately does NOT open the valve — that is a separate, explicit second step.

{
  "schema": "eflostop.cmd",
  "ver": 1,
  "cmd": "leak_reset",
  "id": "req-reset-001"
}

If there was nothing to clear — no incident, no RMLEAK, no override window — the command still acks ok and emits nothing. The acknowledgement does not distinguish that from a real clear, so treat a following rmleak_cleared event as the evidence something changed.

Acknowledgement on success:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 3600
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-reset-001",
    "cmd": "leak_reset",
    "status": "ok"
  }
}

Rejections. Each is a cmd_ack with status:"error"; the error.detail text below is exact.

| error.detail | When |
| --- | --- |
| A leak is still active. Fix the leak first, or use override to open the valve during a leak. | THREE different causes collapse into this one string: a sensor is still reporting wet (the intended case); the rules engine is not initialised; or an internal 1-second lock timed out. The latter two report a leak when no leak is involved. |

main/iothub/app_iothub.c:701-712; main/rules_engine/rules_engine.c:665-732

## L2 — override_enable

The remote equivalent of the physical long-press at the valve, and the sanctioned way to get water during a live leak: clears the incident, starts a 24-hour window in which auto-close is blocked, clears RMLEAK and opens the valve. All preconditions are checked before anything changes, so the window starts on execution, never on receipt.

{
  "schema": "eflostop.cmd",
  "ver": 1,
  "cmd": "override_enable",
  "id": "req-ovr-001"
}

This command can block for up to 10 seconds while it reconnects to the valve, which is why the app's ack timeout must exceed that. It is idempotent in a useful way: calling it while a window is already open refreshes the expiry to a fresh 24 hours and re-emits water_access_override_enabled rather than erroring.

Acknowledgement on success:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 8200
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-ovr-001",
    "cmd": "override_enable",
    "status": "ok"
  }
}

Rejections. Each is a cmd_ack with status:"error"; the error.detail text below is exact.

| error.detail | When |
| --- | --- |
| No valve is set up for this hub. | The hub is not commissioned, or no valve MAC is configured. Checked first. |
| The valve isn't responding. Check its power and connection, then try again. | The valve was not reachable, and stayed unreachable for a bounded 10-second reconnect attempt. |
| No active leak to override. Use the normal Open Valve control. | There is nothing to override: no window already running, no incident latched, no RMLEAK asserted. Pre-emptive use is rejected. |
| Water detected at the valve. It can't be opened remotely until the valve area is dry. | The valve's own flood probe is wet. This is an absolute floor — no override, remote or physical, opens a valve standing in water. |
| Something went wrong applying the override. Your water state is unchanged. Try again. | Internal failure: the rules engine is not initialised, or a lock timed out. |

main/iothub/app_iothub.c:825-848; main/rules_engine/rules_engine.c:818-891

## L3 — override_cancel

Ends the 24-hour window and re-enables auto-close immediately. If leaks are still active it closes the valve and re-asserts RMLEAK there and then, rather than waiting for the next leak report.

{
  "schema": "eflostop.cmd",
  "ver": 1,
  "cmd": "override_cancel",
  "id": "req-ovrc-001"
}

With no window running it acks ok, emits nothing and changes nothing. The window is otherwise time-based: it is not lifted when sensors go dry, only by expiry, this command, or leak_reset.

Acknowledgement on success:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 9000
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-ovrc-001",
    "cmd": "override_cancel",
    "status": "ok"
  }
}

Rejections. Each is a cmd_ack with status:"error"; the error.detail text below is exact.

| error.detail | When |
| --- | --- |
| override cancel failed | Internal failure only — the rules engine is not initialised, or a lock timed out. Cancelling when there is no window is NOT an error. |

main/iothub/app_iothub.c:815-821; main/rules_engine/rules_engine.c:734-816

# Commissioning

provision is additive per key but whole-list-replace per array: sending ble_leak_sensors replaces the entire list, and sending it empty wipes it. decommission with target all is the only command in this document that reboots the hub.

## P1 — provision

Sets the valve, the sensor lists, the auto-close rules and optionally per-sensor metadata, then persists and marks the hub commissioned. Does not reboot. Each key is optional, but at least one of valve_mac, lora_sensors, ble_leak_sensors or rules must be present and well-formed.

{
  "schema": "eflostop.cmd",
  "ver": 1,
  "cmd": "provision",
  "id": "req-prov-001",
  "payload": {
    "valve_mac": "C4:19:D1:88:2A:7F",
    "ble_leak_sensors": [
      "00:80:E1:27:9A:E6",
      "00:80:E1:27:A1:04"
    ],
    "rules": {
      "auto_close_enabled": true,
      "trigger_mask": 7
    },
    "sensor_meta": [
      {
        "sensor_type": "ble_leak_sensor",
        "sensor_id": "00:80:E1:27:9A:E6",
        "location_code": "kitchen",
        "label": "Under sink"
      },
      {
        "sensor_type": "ble_leak_sensor",
        "sensor_id": "00:80:E1:27:A1:04",
        "location_code": "laundry",
        "label": "Behind machine"
      }
    ]
  }
}

| Key | Type | Required | Rules |
| --- | --- | --- | --- |
| valve_mac | string | no | Exactly 17 characters, XX:XX:XX:XX:XX:XX. An invalid value fails the WHOLE command and nothing is saved — unlike a bad sensor MAC, which is merely skipped. |
| lora_sensors | string[] | no | Whole-list replace. Each entry MUST carry the "0x" prefix or it is silently dropped while the command still acks ok (Trap 4). Max 16; a longer list is truncated, not rejected. An empty array wipes the list. |
| ble_leak_sensors | string[] | no | Whole-list replace. Each entry must be a valid 17-character MAC; invalid entries are skipped without failing the command. Max 16, truncated if longer. An empty array wipes the list. |
| rules | object | no | Merged. Accepts auto_close_enabled (boolean) and trigger_mask (number). The three convenience booleans of rules_config are NOT honoured here. |
| sensor_meta | object[] | no | Applied after commissioning succeeds. Elements use exactly the same rules as the standalone sensor_meta command. A rejected element is skipped and never fails the provision. IMPORTANT: this key alone does not satisfy the at-least-one requirement (Trap 3). |

After a successful provision the hub opens a commissioning window: it publishes a commission snapshot once every commissioned device has been heard, or at a 150-second deadline, then keeps refreshing for about six minutes as late devices appear. Storage limits: 16 LoRa sensors, 16 BLE leak sensors, 32 metadata entries in total.

Acknowledgement on success:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 95
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-prov-001",
    "cmd": "provision",
    "status": "ok"
  }
}

Rejections. Each is a cmd_ack with status:"error"; the error.detail text below is exact.

| error.detail | When |
| --- | --- |
| provisioning failed | One string for every cause: no payload; payload not valid JSON; valve_mac present but malformed; none of valve_mac / lora_sensors / ble_leak_sensors / rules present and well-typed; or the storage write failed. |

main/iothub/app_iothub.c:874-906; main/provisioning_manager/provisioning_manager.c:359-526

## P2 — decommission

Removes a single commissioned device, or erases everything and reboots. Removing a sensor also removes its stored location metadata.

{
  "schema": "eflostop.cmd",
  "ver": 1,
  "cmd": "decommission",
  "id": "req-dec-001",
  "payload": {
    "target": "ble_leak_sensor",
    "sensor_id": "00:80:E1:27:A1:04"
  }
}

| Key | Type | Required | Rules |
| --- | --- | --- | --- |
| target | string | yes | One of "valve", "lora", "ble_leak_sensor" or "all". BEWARE the asymmetry: the BLE family also accepts "ble_leak" and "ble" and is case-INSENSITIVE, while "valve", "lora" and "all" are matched case-SENSITIVELY (Trap 5). |
| sensor_id | string | conditional | Required for "lora" and for the BLE family; ignored for "valve" and "all". A LoRa id is parsed as hexadecimal with the "0x" prefix optional here. A BLE MAC must be the full 17-character form, matched case-insensitively. |

Target all is the only command here that reboots. The ack is published immediately, then the hub waits for the valve to disconnect, publishes a final snapshot with reason decommission, waits three seconds and restarts. That final snapshot describes the cleared hub — empty device arrays, a disconnected valve and no hub name — not the site you just wiped.

Acknowledgement on success:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 12000
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-dec-001",
    "cmd": "decommission",
    "status": "ok"
  }
}

Rejections. Each is a cmd_ack with status:"error"; the error.detail text below is exact.

| error.detail | When |
| --- | --- |
| missing decommission target | No payload, payload not parseable, or `target` absent or not a string. |
| unknown decommission target | `target` is a string but matches none of the accepted values — including a case-mismatched "ALL", "Valve" or "LORA". |
| valve decommission failed | Internal failure: manager not initialised, lock timeout, or the storage write failed. |
| lora sensor decommission failed | sensor_id absent or unparseable, the id is not in the commissioned list, or an internal failure. |
| ble sensor decommission failed | sensor_id absent, malformed, not in the commissioned list, or an internal failure. |
| full decommission failed | target "all" and the erase failed. No reboot happens in this case. |

main/iothub/app_iothub.c:714-813

# Configuration

None of these move the valve or change commissioning. sensor_meta and set_hub_name are cosmetic-but-persistent; rules_config decides whether a leak closes the valve at all.

## C1 — sensor_meta

Sets or merges the location code and label for one already-commissioned sensor, keyed by type and id. The payload IS the metadata object — there is no array wrapper on this command.

{
  "schema": "eflostop.cmd",
  "ver": 1,
  "cmd": "sensor_meta",
  "id": "req-meta-001",
  "payload": {
    "sensor_type": "ble_leak_sensor",
    "sensor_id": "00:80:E1:27:9A:E6",
    "location_code": "kitchen",
    "label": "Under sink"
  }
}

| Key | Type | Required | Rules |
| --- | --- | --- | --- |
| sensor_type | string | yes | Case-insensitive. "ble_leak_sensor" is canonical; "ble_leak" and "ble" are permanent aliases. "lora" for a LoRa sensor. |
| sensor_id | string | yes | The MAC or LoRa id. No format validation is applied here. Truncated to 17 characters. Matched case-insensitively, so either case resolves to the same entry. |
| location_code | string | no | Three-state, deliberately: absent keeps the stored value; a recognised name applies it; an UNRECOGNISED name also keeps the stored value and still acks ok (Trap 6). "unknown" is itself a valid value and the only way to clear a location. |
| label | string | no | Free text, silently truncated to 31 characters — never rejected. An empty string clears the label. |

Accepted location codes: unknown, bathroom, kitchen, laundry, garage, garden, basement, utility, hallway, bedroom, living_room, attic, outdoor. A successful update triggers an immediate snapshot so the app picks up the rename without waiting for the heartbeat.

Acknowledgement on success:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 13000
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-meta-001",
    "cmd": "sensor_meta",
    "status": "ok"
  }
}

Rejections. Each is a cmd_ack with status:"error"; the error.detail text below is exact.

| error.detail | When |
| --- | --- |
| sensor metadata update failed | One string for every cause: no payload; payload not valid JSON or not an object; sensor_type missing or unrecognised; sensor_id missing or empty; the 32-entry table is full; or the storage write failed. An unrecognised location_code does NOT fail the command. |

main/iothub/app_iothub.c:859-872; main/sensor_meta/sensor_meta.c:285-344

## C2 — rules_config

Decides whether a leak closes the valve, and which sensor classes are allowed to trigger that. Merged onto the stored configuration, so an empty object is a valid no-op.

{
  "schema": "eflostop.cmd",
  "ver": 1,
  "cmd": "rules_config",
  "id": "req-rules-001",
  "payload": {
    "auto_close_enabled": true,
    "trigger_mask": 7
  }
}

| Key | Type | Required | Rules |
| --- | --- | --- | --- |
| auto_close_enabled | boolean | no | Must be a real JSON boolean. The number 1 or the string "true" are silently ignored. |
| trigger_mask | number | no | Bit 0 = BLE leak sensors, bit 1 = LoRa, bit 2 = the valve's own flood probe; 7 enables all. NO range validation and no clamping anywhere — bits 3 to 7 are stored, echoed back in the twin, and never consulted. 0 is legal and disables every trigger. |
| trigger_ble_leak | boolean | no | Convenience field for bit 0. Applied AFTER trigger_mask, so it wins over that bit if both are sent. |
| trigger_lora | boolean | no | Convenience field for bit 1, same precedence. |
| trigger_valve_flood | boolean | no | Convenience field for bit 2, same precedence. |

Emits no telemetry event and triggers no immediate snapshot — the new values appear in the twin reported properties on the next publish. The same two settings can also arrive inside a provision payload, but only auto_close_enabled and trigger_mask; the convenience booleans are exclusive to this command.

Acknowledgement on success:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 14000
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-rules-001",
    "cmd": "rules_config",
    "status": "ok"
  }
}

Rejections. Each is a cmd_ack with status:"error"; the error.detail text below is exact.

| error.detail | When |
| --- | --- |
| rules config update failed | No payload, payload not valid JSON, the rules engine is not initialised, or the storage write failed. A nonsensical trigger_mask is NOT an error. |

main/iothub/app_iothub.c:850-857; main/rules_engine/rules_engine.c:568-636

## C3 — set_hub_name

Sets the user-facing hub name that appears as gateway.name in telemetry and as hub_name in the twin. Persists across a Wi-Fi reset; cleared by a full decommission.

{
  "schema": "eflostop.cmd",
  "ver": 1,
  "cmd": "set_hub_name",
  "id": "req-name-001",
  "payload": {
    "name": "Main House"
  }
}

| Key | Type | Required | Rules |
| --- | --- | --- | --- |
| name | string | yes | Maximum 31 characters, counted in BYTES — so a UTF-8 name with accents fits fewer glyphs. 32 or more is REJECTED outright, not truncated, and nothing is written. An empty string is a valid explicit CLEAR. |

A successful call immediately publishes the twin reported properties. Caution: the handler ignores the storage result, so a failed write still acks ok (Trap 7). The same value can also be set through the twin desired property hub_name, which takes the same limit but sends no acknowledgement at all.

Acknowledgement on success:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 15000
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-name-001",
    "cmd": "set_hub_name",
    "status": "ok"
  }
}

Rejections. Each is a cmd_ack with status:"error"; the error.detail text below is exact.

| error.detail | When |
| --- | --- |
| missing 'name' field | No payload, payload not parseable, or `name` absent or not a string. |
| name too long (max 31 chars) | Over 31 bytes. Nothing is written and the previous name is kept. |

main/iothub/app_iothub.c:908-925; main/hub_identity/hub_identity.c:91-137

# Traps and sharp edges

These are behaviours that will cost you time if you meet them in the field rather than here. Each was
confirmed by reading the firmware, not inferred from documentation.

Trap 1 — a wrong schema can still execute the command, and you get no acknowledgement. If schema is
anything other than eflostop.cmd or eflostop.cmd.v1, the envelope parser gives up and the raw text is then
scanned for legacy keywords — case-insensitively, anywhere in the message. So {"schema":"eflostop.cmd.v2",
"cmd":"valve_open","id":"abc"} opens the valve, and because it took the legacy path it sends no ack at all.
The cloud sees a timeout and cannot tell "unsupported schema" from "dead hub", while the physical action has
already happened. This affects valve_open, valve_close, leak_reset, override_cancel and the
decommission family. It does NOT affect valve_set_state, override_enable, provision, sensor_meta,
rules_config or set_hub_name, which have no legacy keyword. Always send the exact schema string.

Trap 2 — the same substring scan reads your whole message. Because the legacy matcher searches the entire
uppercased text, any command that fails envelope parsing and happens to contain VALVE_OPEN, LEAK_RESET,
DECOMMISSION_VALVE, RULES_CONFIG:, SENSOR_META: or OVERRIDE_CANCEL anywhere — including inside a sensor
label — is routed to that command. Avoid putting command names in free-text fields.

Trap 3 — a provision carrying only sensor_meta is rejected. The at-least-one-field requirement is
satisfied only by valve_mac, lora_sensors, ble_leak_sensors or rules. A payload whose sole key is
sensor_meta answers provisioning failed and the metadata is never written. Use the standalone sensor_meta
command to set metadata on its own.

Trap 4 — LoRa ids without the 0x prefix are dropped silently. In a provision payload, ["1A2B3C4D"] is
skipped element by element while the command still acks ok with zero sensors stored. Send ["0x1A2B3C4D"].
Inconsistently, decommission accepts the prefix as optional.

Trap 5 — decommission targets are inconsistently case-sensitive. ble, BLE and Ble_Leak_Sensor all
work, but ALL, Valve and LORA are rejected with unknown decommission target. That includes the
destructive all target, where a case mismatch silently no-ops. Send lower case.

Trap 6 — a misspelt location_code keeps the old location and still acks ok. This is deliberate, so a
typo cannot wipe a stored location, but it means you cannot tell from the ack whether the value was applied.
Send unknown if you intend to clear it.

Trap 7 — set_hub_name acks ok even when the write fails. The handler ignores the storage result. The
only failures that surface are the two argument checks. Confirm through the twin reported hub_name if it
matters.

Trap 8 — keep messages under about 1 KB. The receive buffer is 1024 bytes and the handler does not
reassemble fragments, so a larger command arrives as a truncated fragment, fails to parse and is dropped with
no ack. A provision with 16 sensors and inline metadata can approach this — split it into a provision
followed by individual sensor_meta calls.

Trap 9 — cmd and id are truncated without warning, at 31 and 63 characters. An over-long id comes
back truncated, so exact-match correlation fails and the request looks unanswered.

Trap 10 — one error string can mean several things. A leak is still active… also covers an uninitialised
rules engine and an internal lock timeout, neither of which involves a leak. Treat these strings as
user-facing text, not as machine-readable causes.

# Legacy plain-text commands

Before the JSON envelope the hub accepted bare text, and it still does. These forms are recognised
case-insensitively as substrings of the whole message:

VALVE_OPEN, VALVE_CLOSE, LEAK_RESET, OVERRIDE_CANCEL, DECOMMISSION_VALVE, DECOMMISSION_ALL,
DECOMMISSION (whole message), DECOMMISSION_LORA:<hex>, DECOMMISSION_BLE:<mac>, RULES_CONFIG:{json} and
SENSOR_META:{json}.

None of them produce an acknowledgement, because the ack requires either an envelope or a non-empty id.
DECOMMISSION_LORA or DECOMMISSION_BLE without the colon fails to parse and runs nothing.

There is no legacy form for valve_set_state, override_enable, provision or set_hub_name.

Separately, a bare JSON object with no cmd key is treated as a provision payload. That is how the very first
commissioning message can be sent without an envelope. If the object does contain a string cmd but the schema
is unrecognised, it is dropped silently instead.