# eFloStop II Wi-Fi Hub — Telemetry Message Catalogue

Every message the hub can send, as a real example  ·  v2.0

| Document version | 2.0 (supersedes v1.0, which documented firmware 1.8.0) |
| --- | --- |
| Firmware version | 1.9.0 [CMakeLists.txt:12] |
| Git commit | f852ddf7b4209db1fbd2539babcb791898eed94a |
| Schema | eflostop.v2 |
| Topic | devices/<device_id>/messages/events/ (QoS 1) |
| Message count | 37 distinct messages across 8 families |

This document lists every real telemetry message the eFloStop II Wi-Fi Hub can publish to Azure
IoT Hub. Each entry is a complete message exactly as it appears on the wire: the same keys, in the same order,
that the firmware serializer produces for that situation.

Read these as messages you could have captured, not as templates. The example identifiers, names and readings
are realistic and consistent throughout — one hub, one valve and two leak sensors across the whole document —
so an installation can be followed from message to message. The identifiers and readings are examples; the key
sets, the key order and every enum value are exact.

All of these arrive on one topic, devices/<device_id>/messages/events/, at QoS 1. Three envelope type
values carry everything: lifecycle, snapshot and event. Within event, the data.event field tells you
which of the shapes below you have received.

Two things to know before you build a parser. A conditional key is omitted, not sent as null — except
inside the two sensor arrays, where every key is always present and unknown values are null. And in a
snapshot, a sensor that has gone silent reports "leak_state": false, which is indistinguishable from a
genuine dry reading; see message S3.

Changes in firmware 1.9.0. Three things moved, and there is no compatibility shim — every hub runs the
new shape.

1. Water detection is now one event family. The valve's own flood probe used to emit
   valve_flood_detected / valve_flood_cleared with a payload unlike anything else. It now emits
   leak_detected / leak_cleared, the same as the BLE and LoRa sensors, discriminated by
   data.source_type. One handler covers "water was detected somewhere" — see K1 through K6. Every leak
   event carries the same required core (event, source_type, device_id, leak_state, battery,
   location); source-specific keys follow it.

2. The identity key is device_id, everywhere. It was sensor_id, which was wrong once a valve joined
   the same event family. This applies to leak events, auto_close events, health events and both snapshot
   sensor arrays. The snapshot's data.valve.mac is now data.valve.device_id, and the lifecycle message's
   data.valve_mac is now data.valve_device_id. Inbound commands are unchanged — the C2D
   sensor_meta payload still uses sensor_id.

3. One device vocabulary. data.source_type and the health events' data.dev_type now draw from the
   same three values: valve, ble_leak_sensor, lora. The valve previously appeared as valve_flood in
   auto_close events and valve in health events; both now say valve. The one value outside this set is
   source_type: "reconnect" on the reconnect variant of auto_close (R3), which describes a trigger rather
   than a device.

device_id is always UPPERCASE. A BLE MAC is rendered AA:BB:CC:DD:EE:FF — colon-separated, upper
case — and a LoRa id 0x1A2B3C4D, on every message, regardless of the case the device was commissioned
in. Before 1.9.0 a sensor commissioned in lower case appeared lower case in snapshot arrays and upper
case in events: the same physical device under two values of the key you join on. The hub now normalises
on the way out, so a single exact-match join is safe. Matching case-insensitively anyway costs nothing
and is still the more defensive choice.

# Contents

Update field

Word fills this in when fields are updated: select all (Ctrl+A) then press F9.

# What changed since v1.0

Document v1.0 described firmware 1.8.0; this is v2.0, describing firmware 1.9.0. v1.0 is kept unchanged so the two can be read side by side. Every row below is a breaking change — there is no compatibility shim, and every hub runs the new shape.

|  | v1.0 — firmware 1.8.0 | v2.0 — firmware 1.9.0 |
| --- | --- | --- |
| Water at the valve's own probe | event valve_flood_detected / valve_flood_cleared, its own payload shape (msgs V3, V4) | event leak_detected / leak_cleared with source_type "valve" (msgs K5, K6) |
| Identity key, leak events | data.sensor_id | data.device_id |
| Identity key, snapshot sensor arrays | data.lora_sensors[].sensor_id, data.ble_leak_sensors[].sensor_id | data.lora_sensors[].device_id, data.ble_leak_sensors[].device_id |
| Identity key, snapshot valve object | data.valve.mac | data.valve.device_id |
| Identity key, lifecycle | data.valve_mac | data.valve_device_id |
| Identity key, health events | data.sensor_id | data.device_id |
| Identity key, rules events | data.sensor_id | data.device_id |
| Identity key, twin reported | valve_mac | valve_device_id |
| device_id letter case | as commissioned — lower case in snapshot arrays, UPPER case in events, for the same device | always UPPER case, on every message and every plane |
| source_type for the valve | "valve_flood" on auto_close; the valve had no source_type on its own events | "valve" everywhere, matching health events' dev_type |
| device_id value for the valve | the literal "valve" on auto_close; no identity at all on valve events | the valve's BLE MAC on every message |
| Valve identity on valve_state_changed | absent | source_type "valve" + device_id |
| location on leak events | present for sensors only | present for every source; the valve reports code "unknown" until valve metadata exists |
| leak_state on a valve event | re-read after the event name was chosen — a fast toggle could pair leak_detected with leak_state false | the same value that selected the event name, so the pairing cannot contradict |
| Snapshot after a hub-commanded valve move | could report the PRE-transition valve.state | held until the state lands (bounded ~1.5 s), so it reports the post-transition state |

# Index of messages

| ID | type | data.event | Message |
| --- | --- | --- | --- |
| L1 | lifecycle | online | Hub connected, fully commissioned |
| L2 | lifecycle | online | Hub commissioned, but with no devices |
| L3 | lifecycle | online | Hub reconnected after a crash |
| S1 | snapshot | — | Routine heartbeat, everything healthy |
| S2 | snapshot | — | Heartbeat with the valve disconnected |
| S3 | snapshot | — | Heartbeat with a sensor gone silent — read this one carefully |
| S4 | snapshot | — | Snapshot while a water-access override is running |
| S5 | snapshot | — | First snapshot after boot |
| S6 | snapshot | — | Early snapshot as soon as the valve is ready |
| S7 | snapshot | — | Final snapshot before decommissioning |
| V1 | event | valve_state_changed | Valve opened |
| V2 | event | valve_state_changed | Valve closed and locked after a leak |
| V5 | event | valve_state_changed | Valve opened, firmware revision unknown |
| K1 | event | leak_detected | A leak sensor went wet |
| K2 | event | leak_cleared | A leak sensor went dry |
| K3 | event | leak_detected | A leak sensor with no location assigned |
| K4 | event | leak_detected | A LoRa sensor went wet |
| K5 | event | leak_detected | The valve's own flood probe went wet |
| K6 | event | leak_cleared | The valve's own flood probe went dry |
| R1 | event | auto_close | Hub closed the valve because a sensor reported a leak |
| R2 | event | auto_close | Hub closed the valve because its own flood probe went wet |
| R3 | event | auto_close | Hub closed the valve on reconnect, finding a leak still active |
| R4 | event | auto_close_blocked_override | A leak occurred but auto-close was suppressed |
| R5 | event | water_access_override_enabled | A 24-hour water-access override started |
| R6 | event | water_access_override_expired | The override window elapsed |
| R7 | event | auto_close_reenabled | The override was cancelled by command |
| R8 | event | rmleak_cleared | The leak interlock was cleared |
| R9 | event | rmleak_auto_cleared | The leak interlock cleared itself |
| H1 | event | device_offline | A device stopped responding |
| H2 | event | device_recovered | A device came back |
| H3 | event | device_offline | The valve stopped responding |
| C1 | event | cmd_ack | A command succeeded |
| C2 | event | cmd_ack | A command succeeded, no correlation id supplied |
| C3 | event | cmd_ack | A command was refused |
| C4 | event | cmd_ack | An unrecognised command |
| F1 | event | rules_engine | A rules payload could not be re-parsed |
| F2 | event | health_engine | A health payload could not be re-parsed |

# Lifecycle

## L1 — Hub connected, fully commissioned

Sent on every MQTT connect once the hub is provisioned — including reconnects after a network drop, so this is not once per boot.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 42
  },
  "type": "lifecycle",
  "data": {
    "event": "online",
    "reset_reason": "power_on",
    "provisioned": true,
    "valve_device_id": "C4:19:D1:88:2A:7F",
    "lora_sensor_count": 0,
    "ble_leak_sensor_count": 2,
    "rules": {
      "auto_close_enabled": true,
      "trigger_mask": 7
    }
  }
}

main/telemetry/telemetry_v2.c:363-397; trigger main/iothub/app_iothub.c:1696-1699

## L2 — Hub commissioned, but with no devices

Same trigger as L1, for a hub that IS provisioned but has no valve MAC stored and no sensors — a rules-only or empty-array provision payload. NOTE: a hub that has never been provisioned publishes NOTHING at all, so there is no such thing as an unprovisioned lifecycle message; data.provisioned is true on every one you will receive. The hub name is unset here, so gateway.name is omitted, and with no valve stored data.valve_device_id is omitted too. The rules object is always present — it is not gated on commissioning state and always has defaults.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "fw": "1.9.0",
    "uptime_s": 18
  },
  "type": "lifecycle",
  "data": {
    "event": "online",
    "reset_reason": "power_on",
    "provisioned": true,
    "lora_sensor_count": 0,
    "ble_leak_sensor_count": 0,
    "rules": {
      "auto_close_enabled": true,
      "trigger_mask": 7
    }
  }
}

omissions main/telemetry/telemetry_v2.c:82-83, :373-375; unprovisioned skip main/iothub/app_iothub.c:1679-1693

## L3 — Hub reconnected after a crash

Identical shape to L1; only data.reset_reason differs. The seven possible values are power_on, software, panic, watchdog, brownout, deep_sleep and unknown.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 7
  },
  "type": "lifecycle",
  "data": {
    "event": "online",
    "reset_reason": "panic",
    "provisioned": true,
    "valve_device_id": "C4:19:D1:88:2A:7F",
    "lora_sensor_count": 0,
    "ble_leak_sensor_count": 2,
    "rules": {
      "auto_close_enabled": true,
      "trigger_mask": 7
    }
  }
}

mapping main/telemetry/telemetry_v2.c:128-141

# Snapshot

## S1 — Routine heartbeat, everything healthy

The 300 s heartbeat. This is the most common message the hub sends.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 3600
  },
  "type": "snapshot",
  "data": {
    "reason": "heartbeat",
    "system_health": {
      "rating": "excellent",
      "reason": "All devices healthy"
    },
    "valve": {
      "device_id": "C4:19:D1:88:2A:7F",
      "state": "open",
      "battery": 92,
      "leak_state": false,
      "rmleak": false,
      "connected": true,
      "fw_version": "2.2.0",
      "rating": "excellent",
      "last_seen_age_s": 4
    },
    "lora_sensors": [],
    "ble_leak_sensors": [
      {
        "device_id": "00:80:E1:27:9A:E6",
        "connected": true,
        "rating": "excellent",
        "last_seen_age_s": 12,
        "battery": 87,
        "leak_state": false,
        "rssi": -64,
        "fw_version": "1.1.0",
        "location": {
          "code": "kitchen",
          "label": "Under sink"
        }
      },
      {
        "device_id": "00:80:E1:27:A1:04",
        "connected": true,
        "rating": "excellent",
        "last_seen_age_s": 31,
        "battery": 64,
        "leak_state": false,
        "rssi": -78,
        "fw_version": "1.1.0",
        "location": {
          "code": "laundry",
          "label": "Behind machine"
        }
      }
    ],
    "rules": {
      "auto_close_enabled": true,
      "trigger_mask": 7
    },
    "override_active": false
  }
}

main/telemetry/telemetry_v2.c:401-635; interval main/telemetry/telemetry_v2.h:16

## S2 — Heartbeat with the valve disconnected

The BLE link to the valve is down. The valve object collapses: battery, leak_state, rmleak and fw_version are all omitted, and state carries "disconnected" instead of a position.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 5400
  },
  "type": "snapshot",
  "data": {
    "reason": "heartbeat",
    "system_health": {
      "rating": "critical",
      "reason": "Valve offline"
    },
    "valve": {
      "device_id": "C4:19:D1:88:2A:7F",
      "state": "disconnected",
      "connected": false,
      "rating": "critical",
      "last_seen_age_s": 245
    },
    "lora_sensors": [],
    "ble_leak_sensors": [
      {
        "device_id": "00:80:E1:27:9A:E6",
        "connected": true,
        "rating": "excellent",
        "last_seen_age_s": 12,
        "battery": 87,
        "leak_state": false,
        "rssi": -64,
        "fw_version": "1.1.0",
        "location": {
          "code": "kitchen",
          "label": "Under sink"
        }
      },
      {
        "device_id": "00:80:E1:27:A1:04",
        "connected": true,
        "rating": "excellent",
        "last_seen_age_s": 31,
        "battery": 64,
        "leak_state": false,
        "rssi": -78,
        "fw_version": "1.1.0",
        "location": {
          "code": "laundry",
          "label": "Behind machine"
        }
      }
    ],
    "rules": {
      "auto_close_enabled": true,
      "trigger_mask": 7
    },
    "override_active": false
  }
}

disconnected branch main/telemetry/telemetry_v2.c:472-475

## S3 — Heartbeat with a sensor gone silent — read this one carefully

Sensor B has not been heard for over 10 minutes, so the health engine marks it disconnected and the cache merge is skipped. battery, rssi and fw_version go null, but leak_state is written as a hard-coded false. If that sensor was wet when it went silent, this message says it is dry and the earlier leak_detected event is never retracted. last_seen_age_s keeps its real value and is the only field that contradicts the false.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 9000
  },
  "type": "snapshot",
  "data": {
    "reason": "heartbeat",
    "system_health": {
      "rating": "critical",
      "reason": "1 sensor offline"
    },
    "valve": {
      "device_id": "C4:19:D1:88:2A:7F",
      "state": "open",
      "battery": 92,
      "leak_state": false,
      "rmleak": false,
      "connected": true,
      "fw_version": "2.2.0",
      "rating": "excellent",
      "last_seen_age_s": 4
    },
    "lora_sensors": [],
    "ble_leak_sensors": [
      {
        "device_id": "00:80:E1:27:9A:E6",
        "connected": true,
        "rating": "excellent",
        "last_seen_age_s": 12,
        "battery": 87,
        "leak_state": false,
        "rssi": -64,
        "fw_version": "1.1.0",
        "location": {
          "code": "kitchen",
          "label": "Under sink"
        }
      },
      {
        "device_id": "00:80:E1:27:A1:04",
        "connected": false,
        "rating": "critical",
        "last_seen_age_s": 734,
        "battery": null,
        "leak_state": false,
        "rssi": null,
        "fw_version": null,
        "location": {
          "code": "laundry",
          "label": "Behind machine"
        }
      }
    ],
    "rules": {
      "auto_close_enabled": true,
      "trigger_mask": 7
    },
    "override_active": false
  }
}

placeholder main/telemetry/telemetry_v2.c:592-595; merge gate :571-581; F-01

## S4 — Snapshot while a water-access override is running

A 24-hour override window is open, so two extra keys appear alongside override_active — which is present on EVERY snapshot, carrying false when no window is open. override_remaining_s and expires_ts are the conditional pair.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 12000
  },
  "type": "snapshot",
  "data": {
    "reason": "event",
    "system_health": {
      "rating": "excellent",
      "reason": "All devices healthy"
    },
    "valve": {
      "device_id": "C4:19:D1:88:2A:7F",
      "state": "open",
      "battery": 92,
      "leak_state": false,
      "rmleak": false,
      "connected": true,
      "fw_version": "2.2.0",
      "rating": "excellent",
      "last_seen_age_s": 4
    },
    "lora_sensors": [],
    "ble_leak_sensors": [
      {
        "device_id": "00:80:E1:27:9A:E6",
        "connected": true,
        "rating": "excellent",
        "last_seen_age_s": 12,
        "battery": 87,
        "leak_state": false,
        "rssi": -64,
        "fw_version": "1.1.0",
        "location": {
          "code": "kitchen",
          "label": "Under sink"
        }
      },
      {
        "device_id": "00:80:E1:27:A1:04",
        "connected": true,
        "rating": "excellent",
        "last_seen_age_s": 31,
        "battery": 64,
        "leak_state": false,
        "rssi": -78,
        "fw_version": "1.1.0",
        "location": {
          "code": "laundry",
          "label": "Behind machine"
        }
      }
    ],
    "rules": {
      "auto_close_enabled": true,
      "trigger_mask": 7
    },
    "override_active": true,
    "override_remaining_s": 82740,
    "expires_ts": 1785481140
  }
}

main/telemetry/telemetry_v2.c:621-631; window constant main/rules_engine/rules_engine.c:29

## S5 — First snapshot after boot

Identical shape to S1; only data.reason differs. A snapshot's reason has six possible values: heartbeat, event, commission, boot, fast and decommission. (The value "c2d_command" belongs to a different key — data.reason on the auto_close_reenabled event, message R7 — and can never appear on a snapshot.)

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 12
  },
  "type": "snapshot",
  "data": {
    "reason": "boot",
    "system_health": {
      "rating": "excellent",
      "reason": "All devices healthy"
    },
    "valve": {
      "device_id": "C4:19:D1:88:2A:7F",
      "state": "open",
      "battery": 92,
      "leak_state": false,
      "rmleak": false,
      "connected": true,
      "fw_version": "2.2.0",
      "rating": "excellent",
      "last_seen_age_s": 4
    },
    "lora_sensors": [],
    "ble_leak_sensors": [
      {
        "device_id": "00:80:E1:27:9A:E6",
        "connected": true,
        "rating": "excellent",
        "last_seen_age_s": 12,
        "battery": 87,
        "leak_state": false,
        "rssi": -64,
        "fw_version": "1.1.0",
        "location": {
          "code": "kitchen",
          "label": "Under sink"
        }
      },
      {
        "device_id": "00:80:E1:27:A1:04",
        "connected": true,
        "rating": "excellent",
        "last_seen_age_s": 31,
        "battery": 64,
        "leak_state": false,
        "rssi": -78,
        "fw_version": "1.1.0",
        "location": {
          "code": "laundry",
          "label": "Behind machine"
        }
      }
    ],
    "rules": {
      "auto_close_enabled": true,
      "trigger_mask": 7
    },
    "override_active": false
  }
}

snap_reason_str main/iothub/app_iothub.c:581-591; decommission literal :1572

## S6 — Early snapshot as soon as the valve is ready

reason "fast". Fired the moment the valve's BLE setup completes, deliberately before the sensors have beaconed. IMPORTANT: the arrays are NOT short. Array membership comes from the provisioned device list, not from who has checked in — so a sensor that has never been heard still appears, with connected false, rating "critical", last_seen_age_s null and battery, rssi and fw_version all null. A device is absent from these arrays only if it is not provisioned. Because both sensors are still unheard, system_health is critical here; a later snapshot upgrades it as each sensor first beacons.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 25
  },
  "type": "snapshot",
  "data": {
    "reason": "fast",
    "system_health": {
      "rating": "critical",
      "reason": "2 sensors offline"
    },
    "valve": {
      "device_id": "C4:19:D1:88:2A:7F",
      "state": "open",
      "battery": 92,
      "leak_state": false,
      "rmleak": false,
      "connected": true,
      "fw_version": "2.2.0",
      "rating": "excellent",
      "last_seen_age_s": 4
    },
    "lora_sensors": [],
    "ble_leak_sensors": [
      {
        "device_id": "00:80:E1:27:9A:E6",
        "connected": false,
        "rating": "critical",
        "last_seen_age_s": null,
        "battery": null,
        "leak_state": false,
        "rssi": null,
        "fw_version": null,
        "location": {
          "code": "kitchen",
          "label": "Under sink"
        }
      },
      {
        "device_id": "00:80:E1:27:A1:04",
        "connected": false,
        "rating": "critical",
        "last_seen_age_s": null,
        "battery": null,
        "leak_state": false,
        "rssi": null,
        "fw_version": null,
        "location": {
          "code": "laundry",
          "label": "Behind machine"
        }
      }
    ],
    "rules": {
      "auto_close_enabled": true,
      "trigger_mask": 7
    },
    "override_active": false
  }
}

main/iothub/app_iothub.c:587; membership main/health_engine/health_engine.c:444-490

## S7 — Final snapshot before decommissioning

Published out of band immediately before the hub restarts. This is the last message from this identity — and it is a picture of the CLEARED hub, not of the installation. By the time it is built the command handler has already wiped the provisioning config, cleared the hub name, disconnected the valve and emptied the health table, and the hub has waited for the BLE link to actually drop. So the valve object collapses to two keys, both device arrays are empty, and gateway.name is gone. Do not read this message as a final state report of the site.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "fw": "1.9.0",
    "uptime_s": 60000
  },
  "type": "snapshot",
  "data": {
    "reason": "decommission",
    "system_health": {
      "rating": "excellent",
      "reason": "All devices healthy"
    },
    "valve": {
      "state": "disconnected",
      "connected": false
    },
    "lora_sensors": [],
    "ble_leak_sensors": [],
    "rules": {
      "auto_close_enabled": true,
      "trigger_mask": 7
    },
    "override_active": false
  }
}

main/iothub/app_iothub.c:774-801 (clear + disconnect + reload), :1565-1576 (wait, then publish)

# Valve events

The numbering jumps from V2 to V5 on purpose. V3 and V4 were the valve's own flood-probe events (valve_flood_detected / valve_flood_cleared). In firmware 1.9.0 they became ordinary leak events and moved to K5 and K6 under Leak events. The surviving messages keep their original identifiers so this document can be read side by side with v1.0 — V1 here is the same message as V1 there.

## V1 — Valve opened

A GATT notification whose value differs from the last one seen. Delta-gated, so an unchanged notification produces nothing. Carries the same source_type + device_id identity pair as every other device-originated event.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 4000
  },
  "type": "event",
  "data": {
    "event": "valve_state_changed",
    "source_type": "valve",
    "device_id": "C4:19:D1:88:2A:7F",
    "valve_state": "open",
    "battery": 92,
    "leak_state": false,
    "rmleak": false,
    "fw_version": "2.2.0"
  }
}

main/telemetry/telemetry_v2.c:641-675; gate main/iothub/app_iothub.c:1821-1831

## V2 — Valve closed and locked after a leak

The same shape as V1. rmleak true means the valve is latched and will refuse to open until the interlock is cleared.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 4020
  },
  "type": "event",
  "data": {
    "event": "valve_state_changed",
    "source_type": "valve",
    "device_id": "C4:19:D1:88:2A:7F",
    "valve_state": "closed",
    "battery": 92,
    "leak_state": false,
    "rmleak": true,
    "fw_version": "2.2.0"
  }
}

main/telemetry/telemetry_v2.c:641-675

## V5 — Valve opened, firmware revision unknown

Identical to V1 except data.fw_version is omitted, which happens when the DIS read failed at setup. There is NO null variant on this path — the key is simply absent.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 4700
  },
  "type": "event",
  "data": {
    "event": "valve_state_changed",
    "source_type": "valve",
    "device_id": "C4:19:D1:88:2A:7F",
    "valve_state": "open",
    "battery": 92,
    "leak_state": false,
    "rmleak": false
  }
}

main/telemetry/telemetry_v2.c:669-671

# Leak events

K5 and K6 are new in this group. They are the valve's own flood probe, which in v1.0 appeared separately as V3 and V4 under Valve events with a different payload shape. K1 through K6 now share one required core and differ only by source_type.

## K1 — A leak sensor went wet

The safety-critical message. Emitted when a BLE advertisement reports a leak state different from the cached one. Note the sensor's fields are FLAT on data here, whereas a snapshot nests the same sensor inside ble_leak_sensors[].

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 8000
  },
  "type": "event",
  "data": {
    "event": "leak_detected",
    "source_type": "ble_leak_sensor",
    "device_id": "00:80:E1:27:9A:E6",
    "leak_state": true,
    "battery": 87,
    "location": {
      "code": "kitchen",
      "label": "Under sink"
    },
    "rssi": -64
  }
}

main/telemetry/telemetry_v2.c:677-708; trigger main/iothub/app_iothub.c:1845-1858

## K2 — A leak sensor went dry

The clearing counterpart of K1. leak_state is guaranteed false whenever event is leak_cleared — the firmware passes the same value that selected the event name.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 8600
  },
  "type": "event",
  "data": {
    "event": "leak_cleared",
    "source_type": "ble_leak_sensor",
    "device_id": "00:80:E1:27:9A:E6",
    "leak_state": false,
    "battery": 87,
    "location": {
      "code": "kitchen",
      "label": "Under sink"
    },
    "rssi": -66
  }
}

main/iothub/app_iothub.c:1845-1858

## K3 — A leak sensor with no location assigned

Identical to K1 except the location object falls back. The key is always present on this path; code becomes "unknown" and label an empty string.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 8100
  },
  "type": "event",
  "data": {
    "event": "leak_detected",
    "source_type": "ble_leak_sensor",
    "device_id": "00:80:E1:27:A1:04",
    "leak_state": true,
    "battery": 64,
    "location": {
      "code": "unknown",
      "label": ""
    },
    "rssi": -78
  }
}

fallback main/telemetry/telemetry_v2.c:143-153

## K4 — A LoRa sensor went wet

Same shape as K1 with source_type "lora" and a hex device id. NOT SEEN IN PRACTICE: the LoRa radio is not populated on the current PCBA, so no LoRa sensor can report. Included because the firmware path exists.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 8200
  },
  "type": "event",
  "data": {
    "event": "leak_detected",
    "source_type": "lora",
    "device_id": "0x1A2B3C4D",
    "leak_state": true,
    "battery": 78,
    "location": {
      "code": "basement",
      "label": "Sump"
    },
    "rssi": -95
  }
}

main/iothub/app_iothub.c:1747-1763

## K5 — The valve's own flood probe went wet

The valve has a probe of its own, separate from the leak sensors. Same event name and same required core as K1 — source_type "valve" is the discriminator. device_id is the valve's BLE MAC. The valve carries no rssi: it is a GATT link, not an advertisement. location IS present — it belongs to the required core and is emitted for every source — but sensor_meta cannot address a valve yet, so the valve always reports {"code":"unknown","label":""}. In exchange it carries valve_state, rmleak and fw_version.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 4100
  },
  "type": "event",
  "data": {
    "event": "leak_detected",
    "source_type": "valve",
    "device_id": "C4:19:D1:88:2A:7F",
    "leak_state": true,
    "battery": 92,
    "location": {
      "code": "unknown",
      "label": ""
    },
    "valve_state": "open",
    "rmleak": false,
    "fw_version": "2.2.0"
  }
}

main/iothub/app_iothub.c:1788-1820; builder main/telemetry/telemetry_v2.c:677-708

## K6 — The valve's own flood probe went dry

The clearing counterpart of K5. valve_state and rmleak here reflect the auto-close that K5 triggered: the valve is closed and still latched, and stays latched until the interlock is cleared.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 4600
  },
  "type": "event",
  "data": {
    "event": "leak_cleared",
    "source_type": "valve",
    "device_id": "C4:19:D1:88:2A:7F",
    "leak_state": false,
    "battery": 92,
    "location": {
      "code": "unknown",
      "label": ""
    },
    "valve_state": "closed",
    "rmleak": true,
    "fw_version": "2.2.0"
  }
}

main/iothub/app_iothub.c:1788-1820

# Rules events

## R1 — Hub closed the valve because a sensor reported a leak

The automatic shut-off. rmleak_asserted is always true here — it is a presence marker, not a variable. WATCH THE location KEY: on this event it is present only when the source device has a sensor_meta entry, and is OMITTED ENTIRELY otherwise. That is the opposite of the leak events K1-K6, where location is always present and falls back to {"code":"unknown","label":""}. The same un-located sensor therefore yields a location object on leak_detected and no location key at all on the auto_close for the same incident.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 8005
  },
  "type": "event",
  "data": {
    "event": "auto_close",
    "source_type": "ble_leak_sensor",
    "device_id": "00:80:E1:27:9A:E6",
    "rmleak_asserted": true,
    "location": {
      "code": "kitchen",
      "label": "Under sink"
    }
  }
}

main/rules_engine/rules_engine.c:356-386

## R2 — Hub closed the valve because its own flood probe went wet

Same shape as R1 but with source_type "valve" and NO location object. location is omitted when the source is the valve OR when the source sensor has no sensor_meta entry — see R1. device_id is the valve's MAC, the same value K5 and the snapshot report, so this event joins to the same device record.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 4105
  },
  "type": "event",
  "data": {
    "event": "auto_close",
    "source_type": "valve",
    "device_id": "C4:19:D1:88:2A:7F",
    "rmleak_asserted": true
  }
}

main/rules_engine/rules_engine.c:356-386; identity resolution :289-295

## R3 — Hub closed the valve on reconnect, finding a leak still active

A structurally DIFFERENT auto_close. It carries active_leak_count and no location, and source_type is "reconnect", which is not a member of the leak-source enum. Nothing in the firmware reconciles the two shapes.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 400
  },
  "type": "event",
  "data": {
    "event": "auto_close",
    "source_type": "reconnect",
    "device_id": "00:80:E1:27:9A:E6",
    "rmleak_asserted": true,
    "active_leak_count": 1
  }
}

main/rules_engine/rules_engine.c:953-979

## R4 — A leak occurred but auto-close was suppressed

A user override window is open, so the hub deliberately did not close the valve. override_remaining_s is omitted if the remaining time is negative.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 20000
  },
  "type": "event",
  "data": {
    "event": "auto_close_blocked_override",
    "source_type": "ble_leak_sensor",
    "device_id": "00:80:E1:27:9A:E6",
    "override_remaining_s": 61200
  }
}

main/rules_engine/rules_engine.c:481-508

## R5 — A 24-hour water-access override started

Started either by a long press on the valve (trigger "button") or by the override_enable command (trigger "c2d_command"). remaining_s is always the full window constant, 86400.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 19000
  },
  "type": "event",
  "data": {
    "event": "water_access_override_enabled",
    "trigger": "button",
    "expires_ts": 1785484800,
    "remaining_s": 86400
  }
}

main/rules_engine/rules_engine.c:213-222; constant :29

## R6 — The override window elapsed

auto_close_resumed is NOT a report of what the hub did — it is exactly active_leak_count > 0, sampled before any close is attempted, so it duplicates the field beside it. The hub re-closes only if auto-close is still enabled in the rules config; with auto-close disabled this message still says auto_close_resumed true and no closure happens. Nothing on the wire distinguishes the two outcomes, and the expiry-driven closure emits no separate auto_close event — this message is its only trace.

{
  "schema": "eflostop.v2",
  "ts": 1785484800,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 105400
  },
  "type": "event",
  "data": {
    "event": "water_access_override_expired",
    "auto_close_resumed": true,
    "active_leak_count": 1
  }
}

main/rules_engine/rules_engine.c:1052-1068; re-close on expiry :1072-1094

## R7 — The override was cancelled by command

Sent in response to override_cancel. reason is always "c2d_command" here — note this key carries a completely different vocabulary from data.reason on a snapshot.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 25000
  },
  "type": "event",
  "data": {
    "event": "auto_close_reenabled",
    "previous_remaining_s": 61200,
    "reason": "c2d_command"
  }
}

main/rules_engine/rules_engine.c:749-760

## R8 — The leak interlock was cleared

Sent in response to leak_reset. override_cancelled appears only if an override window was open at the time; otherwise the key is absent and this message is just the event name.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 26000
  },
  "type": "event",
  "data": {
    "event": "rmleak_cleared",
    "override_cancelled": true
  }
}

main/rules_engine/rules_engine.c:710-719

## R9 — The leak interlock cleared itself

Every leak source has been dry for 30 seconds, so the hub released the interlock on its own. This clears the latch but does NOT re-open the valve.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 8700
  },
  "type": "event",
  "data": {
    "event": "rmleak_auto_cleared",
    "clear_after_seconds": 30
  }
}

main/rules_engine/rules_engine.c:1107-1123; constant :18

# Health events

## H1 — A device stopped responding

A peer crossed into the critical health rating. These are the only events carrying data.category "health" — that key is how you recognise them. Critical is only ever reached for connectivity reasons; a low battery gives warning, never critical.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 9100
  },
  "type": "event",
  "data": {
    "category": "health",
    "event": "device_offline",
    "dev_type": "ble_leak_sensor",
    "device_id": "00:80:E1:27:A1:04",
    "rating": "critical",
    "prev_rating": "excellent",
    "battery": 64,
    "rssi": -78,
    "offline_duration_s": 600
  }
}

main/health_engine/health_engine.c:579-610; rating :117-145

## H2 — A device came back

The recovery counterpart of H1.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 9700
  },
  "type": "event",
  "data": {
    "category": "health",
    "event": "device_recovered",
    "dev_type": "ble_leak_sensor",
    "device_id": "00:80:E1:27:A1:04",
    "rating": "excellent",
    "prev_rating": "critical",
    "battery": 64,
    "rssi": -75
  }
}

main/health_engine/health_engine.c:588-590

## H3 — The valve stopped responding

Same shape with dev_type "valve". data.rssi is ALWAYS absent for the valve — the hub never records an RSSI for it — and offline_duration_s is omitted when it would be zero. prev_rating is ALWAYS "warning" for a valve, never "excellent": a valve that drops its BLE link is rated warning for a 3-minute grace period first, and only when that expires is it promoted to critical and this alert emitted. Only sensors jump straight to critical (see H1).

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 5500
  },
  "type": "event",
  "data": {
    "category": "health",
    "event": "device_offline",
    "dev_type": "valve",
    "device_id": "C4:19:D1:88:2A:7F",
    "rating": "critical",
    "prev_rating": "warning",
    "battery": 92,
    "offline_duration_s": 180
  }
}

rssi writers only at main/health_engine/health_engine.c:251, :269; omit-on-zero :603-605; grace :150-153, :289-296; promotion :332-340

# Command acknowledgements

## C1 — A command succeeded

Every enveloped command, or any command carrying a correlation id, produces exactly one of these. data.id echoes the id you sent.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 30000
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-8f21",
    "cmd": "valve_close",
    "status": "ok"
  }
}

main/telemetry/telemetry_v2.c:749-772; gate main/iothub/app_iothub.c:932-935

## C2 — A command succeeded, no correlation id supplied

If the inbound command carried no id, data.id is OMITTED — it is not an empty string. You can then only match the ack by cmd and timing.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 30100
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "cmd": "valve_open",
    "status": "ok"
  }
}

main/telemetry/telemetry_v2.c:759-760

## C3 — A command was refused

On failure an error object appears. Note error.code is NOT an error code — it is the command name repeated. The only real discriminator is the free-text detail.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 30200
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-8f22",
    "cmd": "valve_open",
    "status": "error",
    "error": {
      "code": "valve_open",
      "detail": "Valve is locked after a leak (RMLEAK). Clear it with leak_reset first, or use override to open the valve during a leak."
    }
  }
}

main/telemetry/telemetry_v2.c:763-768; message main/iothub/app_iothub.c:539-540

## C4 — An unrecognised command

Any command name the hub does not know produces this. The detail is a fixed string.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 30300
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-8f23",
    "cmd": "reboot_hub",
    "status": "error",
    "error": {
      "code": "reboot_hub",
      "detail": "unknown command"
    }
  }
}

main/iothub/app_iothub.c:928-935

# Fallback messages

## F1 — A rules payload could not be re-parsed

A defensive path. If the rules engine's own JSON fails to re-parse, the hub sends the raw string instead under a fallback event name. You should never see this; if you do, something is wrong on the device.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 31000
  },
  "type": "event",
  "data": {
    "event": "rules_engine",
    "raw": "{\"event\":\"auto_close\",\"source_type\":\"ble_lea"
  }
}

main/telemetry/telemetry_v2.c:710-728; fallback branch :720-725

## F2 — A health payload could not be re-parsed

The health-engine counterpart of F1, with the same caveat.

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "1.9.0",
    "uptime_s": 31100
  },
  "type": "event",
  "data": {
    "event": "health_engine",
    "raw": "{\"category\":\"health\",\"event\":\"device_off"
  }
}

main/telemetry/telemetry_v2.c:730-747; fallback branch :739-744