# eFloStop II Wi-Fi Hub — Cloud-to-Device Command Catalogue

Every command the cloud can send, as a real example  ·  v2.0

| Document version | 2.0 (supersedes v1.0, which documented firmware 1.9.0) |
| --- | --- |
| Firmware version | 2.1.0 [CMakeLists.txt:12] |
| Git commit | 33f7af69ef2a2604353ee25af32be61cf7f27b35 |
| Schema | eflostop.cmd |
| Direction | cloud → hub (C2D) |
| Transport | devices/<device_id>/messages/devicebound/# (MQTT, QoS 1) |
| Command count | 11 commands across 4 groups |

This document lists every command the cloud can send to the eFloStop II Wi-Fi Hub. Each entry is a
complete message exactly as it goes on the wire, followed by the acknowledgement you get back and every rejection
the hub can answer with. The error text is quoted exactly as the firmware emits it.

Commands arrive as ordinary Azure IoT Hub cloud-to-device messages. There are no direct methods and no
device-twin commands; the twin carries configuration only. The hub subscribes once on connect and dispatches
every message it receives on that topic.

Read the Traps section at the end before you write the client. Several behaviours are surprising enough to
cost a day if you meet them in the field instead of here — in particular, a command with the wrong schema can
still execute while sending you no acknowledgement at all.

# Contents

Update field

Word fills this in when fields are updated: select all (Ctrl+A) then press F9.

# What changed since v1.0

Document v1.0 described firmware 1.9.0; this is v2.0, describing firmware 2.1.0. v1.0 is kept unchanged so the two can be read side by side. Unlike the telemetry plane, the command plane keeps a compatibility path: the one renamed key still accepts its old spelling.

|  | v1.0 — firmware 1.9.0 | v2.0 — firmware 2.1.0 |
| --- | --- | --- |
| Maximum command size | ~1 KB shared with the topic; anything larger was dropped silently, with no ack — a provision with inline sensor_meta broke at about six sensors | 4 KB buffer plus fragment reassembly, so all 16 sensors fit in one provision. Over 8 KB is rejected WITH a cmd_ack error correlated to your id (2.1.0) |
| provision — valve identifier | payload.valve_mac | payload.valve_id (valve_mac still accepted, deprecated, logs a warning) |
| provision — auto-close opt-in | only payload.rules.auto_close_enabled (+ trigger_mask), nested | top-level payload.auto_close_enabled added: true also arms trigger_mask=7 (BLE + LoRa + valve probe), false flips the master flag only. The nested rules object still works and wins when both are sent. |
| provision — a malformed valve key | aborts the whole command | the other spelling is tried first; aborts only if NEITHER yields a valid MAC |
| decommission — target matching | the BLE family case-insensitive; "valve" / "lora" / "all" case-SENSITIVE | every target case-insensitive |
| sensor_meta — sensor_type, sensor_id | unchanged | unchanged — inbound metadata keys were already correct |
| rules_config — trigger_valve_flood | unchanged | unchanged — a config bit name, not a device type |
| Twin reported — valve identifier | valve_device_id | valve_id, always present, null when no valve; valve_mac and valve_device_id are explicitly nulled to delete them from the twin |
| provision — twin reported | refreshed only on the next MQTT reconnect, so it could read stale for hours | republished immediately on a successful provision |
| Device Twin — documented at all | absent from this document; snapshot_interval_s was undocumented | full Device Twin section: both desired properties, all 15 reported properties |
| Twin desired — snapshot_interval_s | RAM only: every reboot silently reverted to 300 s while the twin still advertised your value | persisted to NVS, and a twin GET on every connection reconciles a value changed while the hub was offline |
| Twin desired — a rejected value | silently ignored, invisible to the cloud | previous value kept; reported echoes what is IN FORCE, so desired vs reported disagreeing is the rejection signal |
| Twin reported — snapshot_interval_s | not reported at all | reported unconditionally |
| Snapshot after a command | only sensor_meta, provision and decommission; rules_config produced NOTHING, so the change was invisible until the next heartbeat | every command that succeeds is followed by a snapshot, labelled with the command name |
| rules_config — twin reported | stale until the next MQTT reconnect, so polling the twin to confirm read the OLD auto_close_enabled / trigger_mask | republished immediately after the ack |
| Firmware version | 1.9.0 | 2.0.2 |

# The command envelope

Every command is a JSON object with the same four top-level keys. payload is present only
for commands that take arguments.

Keys are read in the order shown. The parser is strict about schema and cmd and permissive about everything
else — ver in particular is read and then never checked.

{
  "schema": "eflostop.cmd",
  "ver": 1,
  "cmd": "valve_close",
  "id": "req-8f23a1"
}

| Key | Type | Required | Notes |
| --- | --- | --- | --- |
| schema | string | yes | Exactly "eflostop.cmd" (or the accepted legacy "eflostop.cmd.v1"). Compared case-sensitively. Any other value does NOT produce an error — see Trap 1. |
| ver | number | no | Defaults to 1 when absent. Read, logged, and then ignored: there is no range check and nothing branches on it. Any number is accepted. |
| cmd | string | yes | The command name, matched case-sensitively. Truncated to 31 characters without warning; a longer name therefore matches nothing and answers "unknown command". |
| id | string | no | Your correlation id, echoed back on the acknowledgement. Truncated to 63 characters without warning. Omit it and you still get an ack for an enveloped command, but the ack carries no id key at all — it is absent, not empty. |
| payload | object | conditional | Required by the six commands that take arguments; must be absent or ignored for the other five. |

# Acknowledgements

The hub answers with a cmd_ack. This is not a reply channel — it is an ordinary telemetry event
on the normal device-to-cloud topic, so you correlate it by data.id.

An ack is sent when the command arrived as a JSON envelope, or when it carried a non-empty id. A legacy
plain-text command satisfies neither, so it executes silently. data.error.code is the command name rather than
a machine-readable code, so match on data.error.detail if you need to distinguish causes.

One caution on correlation: when you omit id, the ack omits data.id entirely rather than sending an empty
string, so a consumer that requires the key will fail to parse.

Every command that succeeds is followed by a snapshot (2.0.2). It carries data.reason "event" and arrives
within a second or two of the ack, so you never have to infer the resulting state from the acknowledgement
alone — read it from the snapshot. A failed command produces the ack only.

Bursts coalesce: several commands in quick succession yield one snapshot, and no more than one snapshot is
published every 5 seconds, so state may be batched but is never skipped.

Success:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "2.1.0",
    "uptime_s": 3600
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-8f23a1",
    "cmd": "valve_close",
    "status": "ok"
  }
}

Failure:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "2.1.0",
    "uptime_s": 3612
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-9c02b7",
    "cmd": "valve_open",
    "status": "error",
    "error": {
      "code": "valve_open",
      "detail": "Valve is locked after a leak (RMLEAK). Clear it with leak_reset first, or use override to open the valve during a leak."
    }
  }
}

# Index of commands

| ID | cmd | Payload | What it does |
| --- | --- | --- | --- |
| V1 | valve_open | none | Open the valve |
| V2 | valve_close | none | Close the valve |
| V3 | valve_set_state | yes | Set the valve to a given position |
| L1 | leak_reset | none | Clear the leak latch after the leak is fixed |
| L2 | override_enable | none | Start a 24-hour water-access override |
| L3 | override_cancel | none | Cancel the override window early |
| P1 | provision | yes | Commission the hub |
| P2 | decommission | yes | Remove one device, or wipe the hub |
| C1 | sensor_meta | yes | Name a sensor and give it a location |
| C2 | rules_config | yes | Configure auto-close |
| C3 | set_hub_name | yes | Name the hub |

# Valve control

Three commands, two code paths. valve_set_state routes to exactly the same handlers as valve_open and valve_close, so prefer it for new work: one command, one shape, and the state you want is explicit rather than implied by the command name. All three queue a BLE write and return immediately — the ack means accepted, not the valve moved. Watch for the valve_state_changed event to know it actually moved.

## V1 — valve_open

Queues a BLE write. Refused only while the RMLEAK interlock is set. No provisioning check, so an uncommissioned hub accepts it and queues a connect that never matches.

{
  "schema": "eflostop.cmd",
  "ver": 1,
  "cmd": "valve_open",
  "id": "req-open-001"
}

The ack means the write was queued, not that the valve moved — a full command queue is dropped silently and still acks ok. Opening an already-open valve emits no event, because valve_state_changed is delta-gated.

Acknowledgement on success:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "2.1.0",
    "uptime_s": 3600
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-open-001",
    "cmd": "valve_open",
    "status": "ok"
  }
}

Rejections. Each is a cmd_ack with status:"error"; the error.detail text below is exact.

| error.detail | When |
| --- | --- |
| Valve is locked after a leak (RMLEAK). Clear it with leak_reset first, or use override to open the valve during a leak. | The valve's RMLEAK interlock is asserted, i.e. it latched after a leak. Clear it with leak_reset, or use override_enable to get water during a live leak. |

main/iothub/app_iothub.c:670-680; refusal main/iothub/app_iothub.c:537-544

## V2 — valve_close

Queues a BLE write. Unconditional — no rejection path, so it can only ack ok.

{
  "schema": "eflostop.cmd",
  "ver": 1,
  "cmd": "valve_close",
  "id": "req-close-001"
}

Does not touch RMLEAK, the incident latch or an active override window.

Acknowledgement on success:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "2.1.0",
    "uptime_s": 3600
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-close-001",
    "cmd": "valve_close",
    "status": "ok"
  }
}

main/iothub/app_iothub.c:681-686

## V3 — valve_set_state

The unified form, and the one to prefer. Same code path as V1/V2, including the RMLEAK refusal when opening.

{
  "schema": "eflostop.cmd",
  "ver": 1,
  "cmd": "valve_set_state",
  "id": "req-state-001",
  "payload": {
    "state": "closed"
  }
}

| Key | Type | Required | Rules |
| --- | --- | --- | --- |
| state | string | yes | Exactly "open" or "closed", case-sensitive. No aliases, no trimming. "OPEN", "Open", "close" and "shut" are all rejected. |

No legacy plain-text form — which is a safety property: a wrong schema here is rejected outright, where the same mistake on V1/V2 executes silently (Trap 1).

Acknowledgement on success:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "2.1.0",
    "uptime_s": 3600
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-state-001",
    "cmd": "valve_set_state",
    "status": "ok"
  }
}

Rejections. Each is a cmd_ack with status:"error"; the error.detail text below is exact.

| error.detail | When |
| --- | --- |
| missing 'state' field (expected "open" or "closed") | No payload, payload is not valid JSON, `state` is absent, or `state` is not a string. Checked before the value is looked at. |
| Valve is locked after a leak (RMLEAK). Clear it with leak_reset first, or use override to open the valve during a leak. | state is "open" and the RMLEAK interlock is set. Same refusal as V1, but error.code here is "valve_set_state". |
| invalid state value (expected "open" or "closed") | `state` is a string but is neither exactly "open" nor exactly "closed". Checked last. |

main/iothub/app_iothub.c:687-713

# Leak and override

These manage the post-leak interlock. leak_reset is the normal path once the leak is fixed; override_enable is the sanctioned way to get water while a leak is still live. Note that override_enable can block for up to 10 seconds while it reconnects to the valve.

## L1 — leak_reset

Clears the incident latch and the valve's RMLEAK, cancels any override window and restores auto-close. Deliberately does NOT open the valve.

{
  "schema": "eflostop.cmd",
  "ver": 1,
  "cmd": "leak_reset",
  "id": "req-reset-001"
}

With nothing to clear it still acks ok and emits nothing. Treat a following rmleak_cleared event as the evidence that something actually changed.

Acknowledgement on success:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "2.1.0",
    "uptime_s": 3600
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-reset-001",
    "cmd": "leak_reset",
    "status": "ok"
  }
}

Rejections. Each is a cmd_ack with status:"error"; the error.detail text below is exact.

| error.detail | When |
| --- | --- |
| A leak is still active. Fix the leak first, or use override to open the valve during a leak. | THREE different causes collapse into this one string: a sensor is still reporting wet (the intended case); the rules engine is not initialised; or an internal 1-second lock timed out. The latter two report a leak when no leak is involved. |

main/iothub/app_iothub.c:714-726; main/rules_engine/rules_engine.c:731-811

## L2 — override_enable

The sanctioned way to get water during a live leak: clears the incident, blocks auto-close for 24 h, clears RMLEAK and opens the valve. Preconditions are checked first, so the window starts on execution, not on receipt.

{
  "schema": "eflostop.cmd",
  "ver": 1,
  "cmd": "override_enable",
  "id": "req-ovr-001"
}

Can block for up to 10 seconds while it reconnects to the valve — size the app's ack timeout accordingly. Calling it while a window is open refreshes the expiry to a fresh 24 h rather than erroring.

Acknowledgement on success:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "2.1.0",
    "uptime_s": 8200
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-ovr-001",
    "cmd": "override_enable",
    "status": "ok"
  }
}

Rejections. Each is a cmd_ack with status:"error"; the error.detail text below is exact.

| error.detail | When |
| --- | --- |
| No valve is set up for this hub. | The hub is not commissioned, or no valve MAC is configured. Checked first. |
| The valve isn't responding. Check its power and connection, then try again. | The valve was not reachable, and stayed unreachable for a bounded 10-second reconnect attempt. |
| No active leak to override. Use the normal Open Valve control. | There is nothing to override: no window already running, no incident latched, no RMLEAK asserted. Pre-emptive use is rejected. |
| Water detected at the valve. It can't be opened remotely until the valve area is dry. | The valve's own flood probe is wet. This is an absolute floor — no override, remote or physical, opens a valve standing in water. |
| Something went wrong applying the override. Your water state is unchanged. Try again. | Internal failure: the rules engine is not initialised, or a lock timed out. |

main/iothub/app_iothub.c:842-866; main/rules_engine/rules_engine.c:897-970

## L3 — override_cancel

Ends the window and re-enables auto-close. If leaks are still active it closes the valve and re-asserts RMLEAK there and then.

{
  "schema": "eflostop.cmd",
  "ver": 1,
  "cmd": "override_cancel",
  "id": "req-ovrc-001"
}

With no window running it acks ok and changes nothing. The window otherwise ends only by expiry, this command or leak_reset — never because sensors went dry.

Acknowledgement on success:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "2.1.0",
    "uptime_s": 9000
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-ovrc-001",
    "cmd": "override_cancel",
    "status": "ok"
  }
}

Rejections. Each is a cmd_ack with status:"error"; the error.detail text below is exact.

| error.detail | When |
| --- | --- |
| override cancel failed | Internal failure only — the rules engine is not initialised, or a lock timed out. Cancelling when there is no window is NOT an error. |

main/iothub/app_iothub.c:832-841; main/rules_engine/rules_engine.c:813-895

# Commissioning

provision is additive per key but whole-list-replace per array: sending ble_leak_sensors replaces the entire list, and sending it empty wipes it. decommission with target all is the only command in this document that reboots the hub.

## P1 — provision

Sets the valve, the sensor lists, the auto-close choice and optional per-sensor metadata. Does not reboot. At least one of valve_id, lora_sensors, ble_leak_sensors, auto_close_enabled or rules must be present and well-formed.

{
  "schema": "eflostop.cmd",
  "ver": 1,
  "cmd": "provision",
  "id": "req-prov-001",
  "payload": {
    "valve_id": "C4:19:D1:88:2A:7F",
    "ble_leak_sensors": [
      "00:80:E1:27:9A:E6",
      "00:80:E1:27:A1:04"
    ],
    "auto_close_enabled": true,
    "sensor_meta": [
      {
        "sensor_type": "ble_leak_sensor",
        "sensor_id": "00:80:E1:27:9A:E6",
        "location_code": "kitchen",
        "label": "Under sink"
      },
      {
        "sensor_type": "ble_leak_sensor",
        "sensor_id": "00:80:E1:27:A1:04",
        "location_code": "laundry",
        "label": "Behind machine"
      }
    ]
  }
}

| Key | Type | Required | Rules |
| --- | --- | --- | --- |
| valve_id | string | no | Exactly 17 characters, XX:XX:XX:XX:XX:XX. Canonical since firmware 2.0.0 — the same key the hub reports back on the snapshot, the events, the lifecycle message and the twin. If NEITHER this nor valve_mac yields a usable MAC, the WHOLE command fails and nothing is saved — unlike a bad sensor MAC, which is merely skipped. |
| valve_mac | string | no | DEPRECATED alias of valve_id, still accepted so existing app and production-tool builds keep commissioning. Logs a warning. valve_id wins when both are present and valid; if valve_id is present but malformed, valve_mac is still tried before the command is failed. Will be removed — migrate to valve_id. |
| lora_sensors | string[] | no | Whole-list replace. Each entry MUST carry the "0x" prefix or it is silently dropped while the command still acks ok (Trap 4). Max 16; a longer list is truncated, not rejected. An empty array wipes the list. |
| ble_leak_sensors | string[] | no | Whole-list replace. Each entry must be a valid 17-character MAC; invalid entries are skipped without failing the command. Max 16, truncated if longer. An empty array wipes the list. |
| auto_close_enabled | boolean | no | The setup-flow opt-in, answered once while the devices are being added. true sets the master flag AND arms trigger_mask to 7 — BLE, LoRa and the valve's own flood probe. false flips the master flag only and leaves trigger_mask untouched, because the mask is not consulted while the flag is off. Omitted or JSON null leaves the stored setting alone; emit null, never false, for 'the user was not asked'. Must be a real boolean — 1 and "true" are ignored with a warning. NOTE this is NOT how the identically named key behaves in rules_config, where it never touches the mask. |
| rules | object | no | Merged. Accepts auto_close_enabled (boolean) and trigger_mask (number). The three convenience booleans of rules_config are NOT honoured here. Applied AFTER the top-level auto_close_enabled, so specific beats shorthand: {"auto_close_enabled":true,"rules":{"trigger_mask":3}} ends up enabled with the valve probe excluded. |
| sensor_meta | object[] | no | Applied after commissioning succeeds. Elements use exactly the same rules as the standalone sensor_meta command. A rejected element is skipped and never fails the provision. IMPORTANT: this key alone does not satisfy the at-least-one requirement (Trap 3). |

A commissioning window follows: a commission snapshot once every device has been heard, or at a 150 s deadline, then refreshes for about six minutes as late devices appear. Twin reported is republished immediately on success (2.0.2). Limits: 16 LoRa, 16 BLE leak sensors, 32 metadata entries.

Acknowledgement on success:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "2.1.0",
    "uptime_s": 95
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-prov-001",
    "cmd": "provision",
    "status": "ok"
  }
}

Rejections. Each is a cmd_ack with status:"error"; the error.detail text below is exact.

| error.detail | When |
| --- | --- |
| provisioning failed | One string for every cause: no payload; payload not valid JSON; a valve key was offered but no spelling of it yielded a valid MAC; none of valve_id / valve_mac / lora_sensors / ble_leak_sensors / auto_close_enabled / rules present and well-typed; or the storage write failed. |

main/iothub/app_iothub.c:891-933; main/provisioning_manager/provisioning_manager.c:359-616

## P2 — decommission

Removes one commissioned device, or erases everything and reboots. Removing a sensor also drops its stored location metadata.

{
  "schema": "eflostop.cmd",
  "ver": 1,
  "cmd": "decommission",
  "id": "req-dec-001",
  "payload": {
    "target": "ble_leak_sensor",
    "sensor_id": "00:80:E1:27:A1:04"
  }
}

| Key | Type | Required | Rules |
| --- | --- | --- | --- |
| target | string | yes | One of "valve", "lora", "ble_leak_sensor" or "all". The BLE family also accepts the aliases "ble_leak" and "ble". All targets are matched case-INSENSITIVELY as of firmware 2.0.0; before that only the BLE family was, so "ALL" and "LORA" were rejected. |
| sensor_id | string | conditional | Required for "lora" and for the BLE family; ignored for "valve" and "all". A LoRa id is parsed as hexadecimal with the "0x" prefix optional here. A BLE MAC must be the full 17-character form, matched case-insensitively. |

Target all is the only command here that reboots. Its final snapshot describes the CLEARED hub — empty arrays, disconnected valve, no name — not the site you wiped.

Acknowledgement on success:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "2.1.0",
    "uptime_s": 12000
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-dec-001",
    "cmd": "decommission",
    "status": "ok"
  }
}

Rejections. Each is a cmd_ack with status:"error"; the error.detail text below is exact.

| error.detail | When |
| --- | --- |
| missing decommission target | No payload, payload not parseable, or `target` absent or not a string. |
| unknown decommission target | `target` is a string but matches none of the accepted values — including a case-mismatched "ALL", "Valve" or "LORA". |
| valve decommission failed | Internal failure: manager not initialised, lock timeout, or the storage write failed. |
| lora sensor decommission failed | sensor_id absent or unparseable, the id is not in the commissioned list, or an internal failure. |
| ble sensor decommission failed | sensor_id absent, malformed, not in the commissioned list, or an internal failure. |
| full decommission failed | target "all" and the erase failed. No reboot happens in this case. |

main/iothub/app_iothub.c:727-831

# Configuration

None of these move the valve or change commissioning. sensor_meta and set_hub_name are cosmetic-but-persistent; rules_config decides whether a leak closes the valve at all.

## C1 — sensor_meta

Sets the location code and label for one already-commissioned sensor. The payload IS the metadata object — no array wrapper on this command.

{
  "schema": "eflostop.cmd",
  "ver": 1,
  "cmd": "sensor_meta",
  "id": "req-meta-001",
  "payload": {
    "sensor_type": "ble_leak_sensor",
    "sensor_id": "00:80:E1:27:9A:E6",
    "location_code": "kitchen",
    "label": "Under sink"
  }
}

| Key | Type | Required | Rules |
| --- | --- | --- | --- |
| sensor_type | string | yes | Case-insensitive. "ble_leak_sensor" is canonical; "ble_leak" and "ble" are permanent aliases. "lora" for a LoRa sensor. |
| sensor_id | string | yes | The MAC or LoRa id. No format validation is applied here. Truncated to 17 characters. Matched case-insensitively, so either case resolves to the same entry. |
| location_code | string | no | Three-state, deliberately: absent keeps the stored value; a recognised name applies it; an UNRECOGNISED name also keeps the stored value and still acks ok (Trap 6). "unknown" is itself a valid value and the only way to clear a location. |
| label | string | no | Free text, silently truncated to 31 characters — never rejected. An empty string clears the label. |

Accepted codes: unknown, bathroom, kitchen, laundry, garage, garden, basement, utility, hallway, bedroom, living_room, attic, outdoor. A successful update triggers an immediate snapshot.

Acknowledgement on success:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "2.1.0",
    "uptime_s": 13000
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-meta-001",
    "cmd": "sensor_meta",
    "status": "ok"
  }
}

Rejections. Each is a cmd_ack with status:"error"; the error.detail text below is exact.

| error.detail | When |
| --- | --- |
| sensor metadata update failed | One string for every cause: no payload; payload not valid JSON or not an object; sensor_type missing or unrecognised; sensor_id missing or empty; the 32-entry table is full; or the storage write failed. An unrecognised location_code does NOT fail the command. |

main/iothub/app_iothub.c:876-890; main/sensor_meta/sensor_meta.c:285-359

## C2 — rules_config

Decides whether a leak closes the valve, and which sensor classes may trigger it. Merged onto the stored config, so an empty object is a valid no-op.

{
  "schema": "eflostop.cmd",
  "ver": 1,
  "cmd": "rules_config",
  "id": "req-rules-001",
  "payload": {
    "auto_close_enabled": true,
    "trigger_mask": 7
  }
}

| Key | Type | Required | Rules |
| --- | --- | --- | --- |
| auto_close_enabled | boolean | no | A PURE master switch here: it never touches trigger_mask. That is deliberately different from the identically named key at the top level of a provision payload, where true also arms all three trigger bits — this command edits settings and changes exactly what you send, provision answers a setup question. Must be a real JSON boolean. The number 1 or the string "true" are silently ignored. |
| trigger_mask | number | no | Bit 0 = BLE leak sensors, bit 1 = LoRa, bit 2 = the valve's own flood probe; 7 enables all. NO range validation and no clamping anywhere — bits 3 to 7 are stored, echoed back in the twin, and never consulted. 0 is legal and disables every trigger. |
| trigger_ble_leak | boolean | no | Convenience field for bit 0. Applied AFTER trigger_mask, so it wins over that bit if both are sent. |
| trigger_lora | boolean | no | Convenience field for bit 1, same precedence. |
| trigger_valve_flood | boolean | no | Convenience field for bit 2, same precedence. |

Emits no telemetry EVENT, but from 2.0.2 a snapshot and a twin reported publish both follow the ack — read the new values from data.rules or from reported. Before 2.0.2 this command produced neither, so the change stayed invisible until the next heartbeat. The same settings also arrive via provision; the convenience booleans are exclusive to this command.

Acknowledgement on success:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "2.1.0",
    "uptime_s": 14000
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-rules-001",
    "cmd": "rules_config",
    "status": "ok"
  }
}

Rejections. Each is a cmd_ack with status:"error"; the error.detail text below is exact.

| error.detail | When |
| --- | --- |
| rules config update failed | No payload, payload not valid JSON, the rules engine is not initialised, or the storage write failed. A nonsensical trigger_mask is NOT an error. |

main/iothub/app_iothub.c:867-875; main/rules_engine/rules_engine.c:634-702

## C3 — set_hub_name

Sets the name shown as gateway.name in telemetry and hub_name in the twin. Survives a Wi-Fi reset; cleared by a full decommission.

{
  "schema": "eflostop.cmd",
  "ver": 1,
  "cmd": "set_hub_name",
  "id": "req-name-001",
  "payload": {
    "name": "Main House"
  }
}

| Key | Type | Required | Rules |
| --- | --- | --- | --- |
| name | string | yes | Maximum 31 characters, counted in BYTES — so a UTF-8 name with accents fits fewer glyphs. 32 or more is REJECTED outright, not truncated, and nothing is written. An empty string is a valid explicit CLEAR. |

Publishes twin reported immediately. The handler ignores the storage result, so a failed write still acks ok (Trap 7). Also settable as a twin desired property, which sends no acknowledgement at all.

Acknowledgement on success:

{
  "schema": "eflostop.v2",
  "ts": 1785398400,
  "gateway": {
    "id": "GW-A0B7651C2D3E",
    "short_id": "2D3E",
    "name": "Main House",
    "fw": "2.1.0",
    "uptime_s": 15000
  },
  "type": "event",
  "data": {
    "event": "cmd_ack",
    "id": "req-name-001",
    "cmd": "set_hub_name",
    "status": "ok"
  }
}

Rejections. Each is a cmd_ack with status:"error"; the error.detail text below is exact.

| error.detail | When |
| --- | --- |
| missing 'name' field | No payload, payload not parseable, or `name` absent or not a string. |
| name too long (max 31 chars) | Over 31 bytes. Nothing is written and the previous name is kept. |

main/iothub/app_iothub.c:934-951; main/hub_identity/hub_identity.c:91-137

# Device Twin

Two settings are changed through the twin rather than through a command. Write them under
properties.desired.

Desired properties produce no cmd_ack. The acknowledgement is that the hub echoes the value it actually
applied into properties.reported — agreement means accepted, disagreement means rejected.

Both persist across a reboot and a Wi-Fi reset, and are cleared only by decommission with target all.

## Desired properties (cloud writes, hub reads)

| Property | Type | Range | Notes |
| --- | --- | --- | --- |
| snapshot_interval_s | number | 60 - 3600 | Heartbeat snapshot cadence in seconds; default 300. Applied immediately, without a reboot or reconnect: the pending deadline is re-aimed from the last confirmed publish, so shortening the interval takes effect at once rather than after the old one expires. A value outside the range is REJECTED with the previous value kept in both RAM and flash - watch for the desired/reported disagreement. Must be a JSON number. |
| hub_name | string | 0 - 31 chars | User-assigned friendly name. Appears as gateway.name in telemetry and hub_name in reported. An empty string clears it. A longer value is ignored with a warning. The set_hub_name command (C3) does the same thing and is preferable when you want an ack. |

{
  "properties": {
    "desired": {
      "snapshot_interval_s": 900,
      "hub_name": "Beach House"
    }
  }
}

## Reported properties (hub writes, cloud reads)

Republished on every MQTT connect, after any desired change, and after a successful provision or
set_hub_name. Written in the order below.

Device-owned: writing any of these under desired does nothing. In particular auto_close_enabled and
trigger_mask are read-only here - change them with rules_config (C2) or the provision opt-in (P1).

| Property | Type | Notes |
| --- | --- | --- |
| fw_version | string | Same runtime value as gateway.fw, from PROJECT_VER. |
| gateway_id | string | GW-xxxxxxxxxxxx, derived from the Wi-Fi MAC. Immutable. |
| short_id | string | Last four hex of the gateway id; also the AP SSID suffix. |
| hub_name | string | Empty string when never set - not absent. |
| provisioned | boolean | Whether any device identity is stored. |
| valve_mac | null | ALWAYS null. Explicitly nulled to DELETE the pre-1.8.0 spelling from the twin. |
| valve_device_id | null | ALWAYS null. Same, for the 1.8.0-1.9.0 spelling. See Trap 12. |
| valve_id | string¦null | Canonical valve identity. Always present, null when no valve is provisioned. |
| lora_sensor_count | number | 0 - 16. |
| ble_leak_sensor_count | number | 0 - 16. |
| auto_close_enabled | boolean | Read-only mirror of the rules config. Omitted if the config read fails. |
| trigger_mask | number | Read-only mirror. Bit 0 BLE, bit 1 LoRa, bit 2 valve probe. |
| uptime_s | number | Seconds since boot at the moment the patch was built. |
| snapshot_interval_s | number | The cadence IN FORCE - not necessarily what desired asks for. This is how you confirm a write landed, and how you detect a rejected one. Added in 2.0.2. |
| free_heap | number | Bytes. Diagnostic only; varies constantly. |

A value set while the hub was offline is not lost. IoT Hub pushes a desired PATCH only on change and
only to a connected device, so the hub also issues a $iothub/twin/GET once per connection and applies the
full document. Changed in 2.0.2 — before that a reboot silently reverted to the 300 s default while the twin
went on advertising your value, with nothing to reconcile the two.

A rejected write looks like desired.snapshot_interval_s: 30 alongside reported.snapshot_interval_s: 300.
Nothing is surfaced to the cloud beyond that disagreement.

Tags are never read by the hub. tags.DeviceModel and anything else there is cloud-side only.

# Traps and sharp edges

These are behaviours that will cost you time if you meet them in the field rather than here. Each was
confirmed by reading the firmware, not inferred from documentation.

Trap 1 — a wrong schema can still execute the command, and you get no acknowledgement. If schema is
anything other than eflostop.cmd or eflostop.cmd.v1, the envelope parser gives up and the raw text is then
scanned for legacy keywords — case-insensitively, anywhere in the message. So {"schema":"eflostop.cmd.v2",
"cmd":"valve_open","id":"abc"} opens the valve, and because it took the legacy path it sends no ack at all.
The cloud sees a timeout and cannot tell "unsupported schema" from "dead hub", while the physical action has
already happened. This affects valve_open, valve_close, leak_reset, override_cancel and the
decommission family. It does NOT affect valve_set_state, override_enable, provision, sensor_meta,
rules_config or set_hub_name, which have no legacy keyword. Always send the exact schema string.

Trap 2 — the same substring scan reads your whole message. Because the legacy matcher searches the entire
uppercased text, any command that fails envelope parsing and happens to contain VALVE_OPEN, LEAK_RESET,
DECOMMISSION_VALVE, RULES_CONFIG:, SENSOR_META: or OVERRIDE_CANCEL anywhere — including inside a sensor
label — is routed to that command. Avoid putting command names in free-text fields.

Trap 3 — a provision carrying only sensor_meta is rejected. The at-least-one-field requirement is
satisfied only by valve_id (or its deprecated alias valve_mac), lora_sensors, ble_leak_sensors,
auto_close_enabled or rules. A payload whose sole key is sensor_meta answers provisioning failed and
the metadata is never written. Use the standalone sensor_meta command to set metadata on its own. The
inverse also bites: auto_close_enabled DOES satisfy the requirement, so a payload whose sole key is that
flag marks an unprovisioned hub as commissioned with no devices. Send it with the devices it describes.

Trap 4 — LoRa ids without the 0x prefix are dropped silently. In a provision payload, ["1A2B3C4D"] is
skipped element by element while the command still acks ok with zero sensors stored. Send ["0x1A2B3C4D"].
Inconsistently, decommission accepts the prefix as optional.

Trap 5 — FIXED in 2.0.0, but check which firmware you are talking to. Up to 1.9.0 the decommission targets
were inconsistently case-sensitive: ble, BLE and Ble_Leak_Sensor all worked, but ALL, Valve and
LORA were rejected with unknown decommission target — the destructive all target being one of the strict
ones, so a case mismatch silently no-opped. From 2.0.0 every target is matched case-insensitively. Sending
lower case remains the safe habit, and is the only thing that works on both.

Trap 6 — a misspelt location_code keeps the old location and still acks ok. This is deliberate, so a
typo cannot wipe a stored location, but it means you cannot tell from the ack whether the value was applied.
Send unknown if you intend to clear it.

Trap 7 — set_hub_name acks ok even when the write fails. The handler ignores the storage result. The
only failures that surface are the two argument checks. Confirm through the twin reported hub_name if it
matters.

Trap 8 — message size: fixed in 2.1.0, but know the ceiling. Up to 2.0.2 the receive buffer was 1024
bytes and the handler did not reassemble fragments, so anything larger arrived truncated, failed to parse and
was dropped with no ack — indistinguishable, from your side, from a message that never arrived. Because
the whole 1024 covered the topic as well as the payload, and inline sensor_meta costs roughly 135 bytes per
sensor, a provision broke at about the sixth sensor.

Since 2.1.0 the buffer is 4096 and oversized messages are reassembled from their fragments, so a provision
carrying all 16 sensors with full-length labels arrives intact. The remaining limit is 8192 bytes, above
which the message is rejected — and rejection is now reported: you get a cmd_ack with
status: "error" and a payload too large detail, correlated against your id, which is recovered from the
first fragment. Splitting a large provision into a provision followed by individual sensor_meta calls is
still the lightest option, and remains the only one that works against a hub running 2.0.2 or earlier.

Trap 9 — cmd and id are truncated without warning, at 31 and 63 characters. GUIDs are unaffected
— a canonical 36-character GUID, or 38 with braces, fits with room to spare, and the ack echoes whatever was
stored without truncating it again. The longest command name the firmware defines is 15 characters. This only
bites a correlation scheme that exceeds 63 characters, where the truncated id comes back unmatched and the
request looks unanswered.

Trap 10 — one error string can mean several things. A leak is still active… also covers an uninitialised
rules engine and an internal lock timeout, neither of which involves a leak. Treat these strings as
user-facing text, not as machine-readable causes.

Trap 11 — a provision whose only valve key is JSON null is fine; one whose value is a malformed string is
not. {"valve_id": null, "ble_leak_sensors":[…]} means "no valve in this payload" and commissions the
sensors normally. {"valve_id": "00:80:E1"} fails the WHOLE command, and the sensors in the same message are
discarded with it. If you serialise optional fields, emit null or omit the key — never an empty or partial
string.

Trap 12 — the device twin keeps deleted properties unless they are explicitly nulled. A reported PATCH is
a merge, so a property the firmware stops writing freezes at its last value rather than disappearing. The
valve identity has been renamed twice, so 2.0.0 sends "valve_mac": null and "valve_device_id": null to
delete the old spellings. Read reported.valve_id; do not write a ?? fallback chain across the three
names, or you will keep reading a stale value forever.

Trap 13 — re-sending auto_close_enabled: true on an incremental provision silently re-arms every trigger.
The key is the setup-flow opt-in, and true always sets trigger_mask back to 7. A later provision that
merely adds a sensor and repeats the flag out of habit will undo any per-source narrowing the user made
through rules_config in between, with no warning and an ok ack. Send the key only on the commissioning
message where the user actually answered the question; omit it (or send JSON null) on every other
provision.

# Legacy plain-text commands

Before the JSON envelope the hub accepted bare text, and it still does. These forms are recognised
case-insensitively as substrings of the whole message:

VALVE_OPEN, VALVE_CLOSE, LEAK_RESET, OVERRIDE_CANCEL, DECOMMISSION_VALVE, DECOMMISSION_ALL,
DECOMMISSION (whole message), DECOMMISSION_LORA:<hex>, DECOMMISSION_BLE:<mac>, RULES_CONFIG:{json} and
SENSOR_META:{json}.

None of them produce an acknowledgement, because the ack requires either an envelope or a non-empty id.
DECOMMISSION_LORA or DECOMMISSION_BLE without the colon fails to parse and runs nothing.

There is no legacy form for valve_set_state, override_enable, provision or set_hub_name.

Separately, a bare JSON object with no cmd key is treated as a provision payload. That is how the very first
commissioning message can be sent without an envelope. If the object does contain a string cmd but the schema
is unrecognised, it is dropped silently instead.